Experiment 10
Capture and analyse network packets using network Packet Analyzer

What is wireshark doing behind the scenes?

Wireshark uses you Network Interface Card (NIC) in a special mode to see all traffic – not just packets addressed to your device.

Wireshark gains some special permission through the NIC card and works in 2 modes:

1.  Promiscuous Mode:

Normally your NIC ignores other devices packets. Wireshark enables Promiscuous Mode to capture all packets on the network segment.

2. Monitor Mode (Wireless)

On Wi-fi, Monitor Mode captures all wireless frames in the air – even those not meant for your device.

3. Libcap / ncap [Tools and Drivers]

Wireshark relies on libcap(Linux/Mac) or Ncap(Windows) to pull raw packets directly from OS Kernel.

Flow:

PC1, PC2 -> Libcap/Ncap [Fetches data from kernel] -> NIC -> Wireshark – Packet Sniffing

Protocols to capture:

1. HTTP / HTTPS

2. ARP

3. DHCP

4. DNS

5. ICMP

6. SSH / FTP

What are network interfaces?

icmp && ip.addr == 8.8.8.8

ICMP Packet Capture Using Wireshark

Steps:

| Step | What to do |
| --- | --- |
| 1 | Connect your laptop to the Internet using Ethernet, Wi-Fi, or mobile hotspot. |
| 2 | Open Wireshark. |
| 3 | Select your active interface: Ethernet if using LAN cable, Wi-Fi if using hotspot/Wi-Fi. |
| 4 | Double-click the interface to start packet capture. |
| 5 | Keep Wireshark capturing. Do not stop it. |
| 6 | Now open Command Prompt (CMD). |
| 7 | Run ping 8.8.8.8 |
| 8 | Wait until the ping command finishes. |
| 9 | Return to Wireshark. |
| 10 | Click the red Stop button to stop capturing. |
| 11 | Apply the filter icmp && ip.addr == 8.8.8.8 |
| 12 | Identify the Echo Request and corresponding Echo Reply. |

Ethernet Header = 14 bytes

IPv4 Header     = 20 bytes

ICMP Header     =  8 bytes

Ping Data       = 32 bytes

-------------------------

Total           = 74 bytes

ARP Packet Capture Using Wireshark

Steps:

| Step | What to do |
| --- | --- |
| 1 | Connect your laptop to Ethernet/Wi-Fi/mobile hotspot. |
| 2 | Open CMD first. |
| 3 | Run ipconfig and note the Default Gateway IP. |
| 4 | Open Command Prompt as Administrator. |
| 5 | Run arp -d * to clear dynamic ARP cache entries for a clean demonstration. |
| 6 | Now open Wireshark. |
| 7 | Select the active Ethernet or Wi-Fi interface. |
| 8 | Start packet capture. |
| 9 | Keep Wireshark capturing and return to CMD. |
| 10 | Run ping <Default-Gateway-IP> — for example, ping 192.168.0.1. |
| 11 | After ping finishes, return to Wireshark. |
| 12 | Click the red Stop button. |
| 13 | Apply the Wireshark filter arp. |
| 14 | Find the ARP Request and corresponding ARP Reply. |
| 15 | Analyze Source, Destination, Length, Info, Sender IP/MAC and Target IP/MAC. |

DNS Packet Capture Using Wireshark

Steps:

| Step | What to do |
| --- | --- |
| 1 | Connect the laptop to the Internet using Ethernet/Wi-Fi/mobile hotspot. |
| 2 | Open Wireshark first. |
| 3 | Select the active Ethernet or Wi-Fi interface. |
| 4 | Start packet capture. |
| 5 | Keep Wireshark capturing. |
| 6 | Now open CMD. |
| 7 | Run nslookup example.com |
| 8 | Wait for the DNS result to appear in CMD. |
| 9 | Return to Wireshark. |
| 10 | Click the red Stop button. |
| 11 | Apply the filter dns.qry.name == "example.com" |
| 12 | Identify the A Query and its Response. |
| 13 | Analyze Source, Destination, Protocol, Length and Info. |
| 14 | Click the packets and inspect the DNS and UDP fields. |

TCP Packet Capture Using Wireshark

Steps:

| Step | What to do |
| --- | --- |
| 1 | Connect laptop to Internet using Ethernet/Wi-Fi/mobile hotspot. |
| 2 | Open Wireshark first. |
| 3 | Select the active Ethernet or Wi-Fi interface. |
| 4 | Start packet capture. |
| 5 | Keep Wireshark capturing. |
| 6 | Now open CMD. |
| 7 | Run curl http://example.com |
| 8 | Wait until the command finishes and HTML/output appears. |
| 9 | Return to Wireshark. |
| 10 | Click the red Stop button. |
| 11 | Apply filter tcp.port == 80 |
| 12 | Find the TCP connection generated for example.com. |
| 13 | Identify [SYN] → [SYN, ACK] → [ACK]. |
| 14 | Examine Source, Destination, Ports, Sequence/Acknowledgment numbers and Flags. |
| 15 | Optionally use Follow → TCP Stream to show HTTP GET and 200 OK. |